# BIMDesigner

Technology, progress and the MEP roadmap

Status report · 22 October 2026 · Version 0.5.0

## 1. Summary

BIMDesigner is a building information modelling (BIM) application for Windows. A building is modelled once – walls, floors, roofs, doors, windows, rooms, structure and fittings, each with its real build-up – and every plan, section, elevation, 3D view, schedule and drawing sheet is drawn from that one model. While the model is drawn, it is checked against the UK Building Regulations and British Standards, or the US building codes. It exchanges models with other BIM programs through IFC, the open industry standard.

Work started on 21 September 2026. Twelve rounds of work since then have taken it from an empty project to complete architectural design and documentation, with structure, coordination, programme and cost on top. The next major step is MEP – mechanical, electrical and plumbing services – set out in section 4.

| 137,000 lines of application code, plus 48,000 lines of tests | 1,889 automated tests, all passing | 12 rounds of work completed and released | 266 IFC files from other programs opened and checked |
| --- | --- | --- | --- |

## 2. Our stack

| Part | Technology | What it gives us |
| --- | --- | --- |
| Language and runtime | C# on .NET 10, a long-term support release | One language across the whole product; fast and memory-safe; supported by Microsoft until November 2028. |
| Modelling engine | Our own, in C# (about 74,000 lines). No third-party geometry kernel, and not tied to Windows | Walls and their joins, roofs, rooms, structure, regulation checks, schedules, clashes, programme and cost. It can later run behind a web viewer or on a server. |
| Desktop application | WPF on Windows (about 50,000 lines) | A native Windows interface with a ribbon, and hardware-accelerated plan and 3D views. |
| Open exchange | Xbim.Essentials 6.1 for IFC2x3 and IFC4, with our own reader for IFC geometry; our own Excel writer | IFC in and out, COBie 2.4 handover data, BCF 2.1 issues, Microsoft Project XML programmes and Excel reports. |
| Drawings | Our own vector PDF writer; netDxf for DXF | Sheets and plans printed at true scale, and sent to CAD programs. |
| Project files | .bimx, a structured JSON format | Readable and versionable; a recovery copy is kept every five minutes and after any error. |
| Quality | xUnit: 1,889 tests, including tests that drive the real application window | Every round is tested before it is released; the whole suite runs in about two minutes. |
| Release | WiX Toolset 5 MSI installer; release script with code signing; Git on GitHub | Installs for every user of a PC and opens .bimx files on double-click. Signing is ready and waits only on a certificate. |

The code is in four parts: the engine, the file formats, the application and the tests. The engine knows nothing about the screen, which is why it can be tested thoroughly and reused when the product grows beyond the desktop.

## 3. What we have achieved

Each round takes one area and finishes it. First we research how it really works – the regulations, the standards and how designers use it. Then we build it, test it and document it.

| Area | In place today |
| --- | --- |
| Building model | Layered walls – straight, curved, spline, slanted and stacked – and curtain walls with mullion types checked for wind. Floors, ceilings and roofs of every common shape, with eaves, gutters, soffits, dormers and roof windows. Doors, windows, stairs, ramps, railings and rooms. Site, terrain and property lines. Massing and building by face. |
| Structure | Beams and joists. Structural columns in steel, timber, glulam and concrete. Strip and pad footings sized for the loads taken down from the model. |
| Regulation checks | The model is checked continuously against Approved Documents A, B, E, F, H, J, K, L and M, the Nationally Described Space Standard and related British Standards – or the IRC and ADA in the US. Problems are shown room by room and drawn in red. U-values are worked out from each build-up’s layers (BS EN ISO 6946). |
| Views and documents | Plans, sections, elevations, interior elevations, callouts and named 3D views. Dimensions, tags, keynotes, revisions and revision clouds. Schedules with formulas, and sheets with title blocks. Phasing, view filters, design options and view templates. |
| Coordination (4D, 5D) | Clash detection, with each clash tracked as an issue from run to run and reported as BCF. A construction programme with a critical path, played day by day in 3D. A cost plan to NRM1 or UniFormat II, with cash flow and earned value. |
| People in the model | Walking through the model as an adult, a child or a wheelchair user, each stopped where they would be. What a person can see is shaded on the plan, and the view out of each room is graded to BS EN 17037. |
| Exchange and handover | IFC2x3 and IFC4 import and export, COBie 2.4, Uniclass classification, DXF, PDF, and programmes for Microsoft Project, Primavera P6 and Asta. |
| Hardening (round 12) | On a 2,883-element building, the plan redraws in 0.6 s (it took 9–10 s) and the 3D model builds in 0.9 s (it took 8 s). Errors are logged and recovered from instead of closing the program, and unsaved work is offered back after a crash. Real IFC files from other programs now open correctly. A first-hour usability pass, drawing a house from scratch, found and fixed what got in a user’s way. |

Not yet built: MEP services (section 4); collaboration – several people on one model, linked models, a web viewer (Phase 3); rebar and the structural analysis model; rendering with textures; code-signed releases.

## 4. MEP roadmap

MEP covers heating, ventilation, water, drainage, power, lighting and fire protection. It is the largest area of our original specification still to be built (Phase 4). BIMDesigner does not start from zero:

- Sanitaryware, radiators, lights, sockets, switches, extract fans, consumer units and smoke, heat and CO alarms are already placed and checked against Approved Documents F, L and M, BS 5839-6 and the US NEC.

- Gutters and downpipes are sized to BS EN 12056-3.

- Every build-up’s U-value is already known.

- The clash, programme, cost and IFC engines are ready to take services.

What is missing are the services themselves – the pipes, ducts and cables that join these fittings into systems – and the calculations that size them. We will build MEP in rounds, the same way as before. We start with homes, where the regulations are already modelled, and then extend to commercial buildings.

| Round | Scope | Designed and checked to | Outcome |
| --- | --- | --- | --- |
| M1 Connections and systems | Connection points on every fitting and piece of equipment, with flow direction. Systems for cold water, hot water, heating, foul water, rainwater, supply and extract air, power and lighting. A system browser and warnings for anything left unconnected. MEP spaces and zones taken from rooms. Discipline plan views, with the architecture greyed underneath. | IFC4 distribution systems and ports | The model knows what is connected to what. |
| M2 Pipes and water | Pipe types by material (copper, PEX, multilayer, plastic, steel), drawn in plan, section and 3D with their fittings made automatically. Valves, pumps and cylinders. Hot and cold water sized from loading units. Hot-water dead legs and water use. | BS EN 806-3, BS 8558, Approved Document G; IPC water supply fixture units | A home’s water supply designed and sized. |
| M3 Drainage | Soil, waste and vent pipes laid at their falls, with traps, stacks, branches and air admission valves. Below-ground drains with manholes, inspection chambers, invert levels and long sections. Rainwater joined to the existing downpipes. | BS EN 12056-2, BS EN 752, Approved Document H; IPC drainage fixture units | Drainage above and below ground, drawn and checked. |
| M4 Heating | Room-by-room heat loss from the model’s own U-values. Radiators and underfloor loops sized to it. Boilers, heat pumps and cylinders. Heating pipework sized for flow and pressure drop. | BS EN 12831-1, MCS MIS 3005-D, BS EN 1264, CIBSE Guide C; ACCA Manual J | Heating designed from the building’s own heat loss. |
| M5 Ventilation and ducts | Round, rectangular, oval and flexible ducts with fittings. Grilles, diffusers, fans and heat-recovery (MVHR) units. Airflow for each room, duct sizing and a pressure-loss report. Fire dampers where ducts cross fire-resisting walls and floors. | Approved Documents F and B, BS EN 16798, CIBSE Guide B; ASHRAE 62.2, ACCA Manual D | Ventilation designed and sized. |
| M6 Electrical and lighting | Distribution boards and circuits with their loads, diversity and protective devices. Cables sized for current and voltage drop, with board and circuit schedules. Luminaires with photometric data, and lux levels per room. Switches linked to lights. Conduit and cable tray checked for fill. Data and fire alarm devices. | BS 7671, Approved Document P, BS EN 12464-1, BS 5839-6; NEC Articles 210, 220 and 310 | Power and lighting designed, scheduled and checked. |
| M7 Fire protection and gas | Sprinkler heads, their coverage and their pipework. Gas pipework sized from each appliance’s demand back to the meter. | BS 9251, BS EN 12845, BS 6891, Approved Document J; NFPA 13D/13R | Sprinkler and gas layouts for homes and small buildings. |
| M8 Coordination and handover | MEP in clash tests against the structure, allowing for insulation and access space. Sleeves and openings cut automatically where services pass through walls and floors. Hangers and supports at their spacings. MEP in IFC4 import and export, COBie, the cost plan (NRM1 group 5) and the programme. gbXML export for energy analysis. | IFC4, COBie 2.4, NRM1, gbXML | Services coordinated, costed and handed over with the rest of the model. |

Milestones

- After M3: a house’s water and drainage, end to end.

- After M5: heating and ventilation designed from the building’s own performance.

- After M6: all three disciplines in the model, enough for a full set of MEP drawings.

- After M8: services in the same coordination, cost and handover workflows as the architecture and structure.

Order. M1 comes first because every later round depends on connections. Water and drainage follow because the sanitaryware and the drainage rules already exist. Heating follows because the U-values already exist. Electrical comes later because it is the largest round.

Later, after M8: commercial plant (air handling units, VAV boxes, chillers), automatic routing of pipes and ducts, fabrication detail (spools and shop drawings) and medical gases.